ProSil SR 108 for MBM , impregnation for Mineral Building Materials.

Material Safety Data Sheet

according to Regulation (EC) 1907/2006

Product name : PROSIL SR 108 FOR MBM

Revision : 01.10.2008 Version : 4.0.0

01. Identification of the substance/preparation and of the company/undertaking

Product name

PROSIL SR 108 FOR MBM

product number 54401801

Manufacturer/Supplier

Reinhard Oil A/S

Street/P.O.Box

Helleruplund  Alle 8

Country code/Postal code/Town/City

DK-2900  Hellerup Denmark Europe

Telephone / Telefax

+45 7026 7007 / +45 7026 7047

Contact

Steen Qvist, chemical engineer  sq@reinhardoil.dkEmergency information

+45 7026 7007 or +45 39626221

02. Hazards identification

Hazard designation

Flammable.

Classification : R 10

Additional information

Inhalation of aerosol spray may damage health. Product hydrolyses under formation of methanol (CAS no. 67-56-1). Methanol is toxic if

inhaled, swallowed or comes into contact with the skin (T, R23/24/25), leads to irreversible damage if inhaled, comes into contact with the

skin or is swallowed (T, 39/23/24/25) and is highly flammable (F, R11).

03. Composition/information on ingredients

Hazardous components

(2,4,4-TRIMETHYLPENTYL)TRIMETHOXYSILANE ; EC-No. : 251-995-5; CAS-No. : 34396-03-7

Percentage : 2,5 - 10 %

Classification : R 10 R 52/53

METHANOL ; EC-No. : 200-659-6; CAS-No. : 67-56-1

Percentage : < 1 %

Classification : F ; R 11 T ; R 39/23/24/25 T ; R 23/24/25

For the wording of the listed risk phrases refer to section 16.

04. First-aid measures

General

In all cases of doubt, or when symptoms persist, seek medical attention. Never give anything by mouth to an unconscious person. For

further information refer to "specifications about toxicology".

After inhalation

Take the casualty into the fresh air and keep warm. Irregular breathing/no breathing: artificial respiration. Unconsciousness: lateral

position - call a physician.

After skin contact

Immediately remove all contaminated clothing. Wash away with soap and water and rinse. Do NOT use solvents or thinners.

After eye contact

Remove contact lenses, keep eyelids open. Flush with plenty of water (10 - 15 min.). Call a physician.

After ingestion

First-aid measures Do not induce vomiting. Rinse mouth with cold water. Keep at rest.

Notes to a physician

In case of contact with water material splits off (also in gastrointestinal tract) methanol in larger amounts; therefore consider poisoning on

methanol and also observe known period of latency of several days!

05. Fire-fighting measures

Suitable extinguishing media

Alcohol resistant foam, CO2, powders, water spray.

Unsuitable extinguishing media

Waterjet.

Special risk posed by the substance or by the actual preparation, its combustion products or gases

discharged

In case of fire do not breathe fumes

Special protective equipment

Appropriate breathing apparatus may be required.

Additional information

Cool endangered containers with water in case of fire. Do not allow the quenching water into the sewage system.

06. Accidental release measures

Personal precautions

Keep away from ignition sources on account of the organic solvent content and air room well. Do not inhale vapours. Refer to protective

measures listed in sections 7 and 8.

Environmental precautions

Do not empty into drains. If the product contaminates lakes, rivers or sewages, inform appropriate authorities in accordance with local

regulations.

Methods for cleaning up/collecting

Contain and collect spillage with non-combustible absorbent materials, e.g. sand, earth, vermiculite, diatomaceous earth and place in

container for disposal according to local regulations (see section 13). Clean preferabily with a detergent; avoid use of solvents.

07. Handling and storage

Information for safe handling

Prevent the creation of inflammable or explosive concentrations of vapour in air and avoid vapour concentrations higher than the OEL

(=Occupational Exposure Limit). Additionally, the product should only be used in areas from which all naked lights and other sources of

ignition have been excluded. Avoid contact with skin and eyes. Do not inhale the vapour. Do not eat or drink during work - no smoking.

Comply with the health and safety at work laws.

Information about protection against explosions and fires

Vapours are heavier than air and may spread along floors. Vapours may form explosive mixtures with air. No sparking tools should be

used.

Requirements to be met by storerooms and containers

Electrical equipment should be pr.tected to the appropriate standard. Floors should be of the conducting type. Keep container tightly

closed. Never use pressure to empty: container is not a pressure vessel. No smoking. Prevent unauthorized access. Containers which are

opened must be carefully resealed and kept upright to prevent leakage.

Information about separation of incompatible products

Keep away from oxidizing agents, from strongly alkaline and strongly acid materials.

Further information about storage conditions

Always keep in containers of same material as the original one. See also instructions on the label. Avoid heating and direct sunlight. Keep

away from ignition sources - No smoking.

Storage class (VCI) : 3

08. Exposure controls/personal protection

Additional information about engineering measures

Provide adequate ventilation. Where reasonably practicable this should be achieved by the use of local exhaust ventilation and good

general extraction. If these are not sufficient to maintain concentrations of particulates and solvent vapour below the OEL (=Occupational

Exposure Limit) , suitable respiratory protection must be worn.

Personal protective equipment

Respiratory protection

If workplace limits are exceeded, a gas mask approved for this purpose must be worn. Fresh air masks are recommended, or

combination filters A2-P2 for works of short duration.

Hand protection

Use protective gloves.

Eye protection

Use tightly fitting safety glasses.

09. Physical and chemical properties

Relevant safety data

Physical state : liquid

Odour : Characteristic.

Colour : Opal.

Boiling point / range : ( 1013 hPa ) > 190 °C

Flash point : 44 °C

Vapour pressure : ( 50 °C ) < 1000 hPa

Density : ( 20 °C ) ca. 1,1 g/cm3

Solubility in water : ( 20 °C ) insoluble

Viscosity : ( 20 °C ) ca. 20 mPa.s

10. Stability and reactivity

Conditions to avoid

moisture

Materials to avoid

Reacts with: water , basic substances and acids . Reaction causes the formation of: methanol .

Hazardous decomposition products

Under the effect of humidity, water and protic agents: methanol . The following applies for the silicone content of the substance:

Measurements have shown the formation of small amounts of formaldehyde at temperatures above about 150 °C (302 °F) through

oxidation.

11. Toxicological information

There are no data available on the preparation itself. The product was classified in toxicological terms on the basis of the results of the

calculation procedure outlined within General Directive on Preparations (1999/45/EC). Prolonged inhalation of vapours in high concentrations

may lead to headache, giddiness and nausea. Inhalation/eye contact: in high concentrations irritating to the mucous membranes, narcotic

effect and influence on power of reaction and loss of coordination possible. Skin and eye contact: frequent and long lasting contact may

cause irritation and skin inflammation. Eye contact: irritation.

Additional toxicological information

Product(s) of hydrolysis: Attention! Product may hydrolyse in gastro-intestinal tract and produce methanol. According to literature

methanol (67-56-1) irritates mucuous membranes, has skin drying and narcotic effects up to coma or death. Absorption by the skin is

possible. Possibility of damage to heart, kidneys, liver and optic nerves (blindness) over a period of time.

12. Ecological information

Do not empty into waters or drains.

13. Disposal considerations

In accordance with local official regulations.

14. Transport information

Land transport ADR/RID

Remarks

Not regulated in Class 3 - ADR/RID 2.2.3.1.1 NOTE 1 - Substance does not sustain combustion!

Maritime transport IMDG/GGVSea

Remarks

Not regulated in Class 3 - IMDG 2.3.1.3 - Substance does not sustain combustion!

Air transport ICAO-TI and IATA-DGR

Remarks

Not regulated in Class 3 - IATA 3.3.1.3 / ICAO 3.1.3 - Substance does not sustain combustion!

15. Regulatory information

Classification according to EC directives

R-phrases

10 Flammable.

S-phrases

35 This material and its container must be disposed of in a safe way.

National regulatory information

Water pollution classification

Class : - according VwVwS

16. Other information

The details in this material safety data sheet satisfy national and EC legislation. We have no knowledge or control over the user´s working

conditions however. The user is responsible for the observance of all required statutory provisions.

Further information

Relevant changes

07. Information about separation of incompatible products . 08. References to the thresholds

R-Phrases of components

10 Flammable.

11 Highly flammable.

23/24/25 Toxic by inhalation, in contact with skin and if swallowed.

39/23/24/25 Toxic: danger of very serious irreversible effects through inhalation, in contact with skin and if swallowed.

52/53 Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

These data are based on our present knowledge. However, they shall not constitute a guarantee for any specific product features and shall not

establish a legally valid contractual relationship.